This document is intended to describe a method for using

Lattice C (small model) to develop Installable Device Drivers for

MS-DOS.  Additionally, a number of C functions (provided in

library format) are defined.

Installable Device Drivers are created by first writing the

functions to be supported, then linking these functions with the

Driver Header.  A prototype header file is provided in the file

HDR.ASM.  Note that two data items (the Attribute and Name/unit

fields) must be filled in with information specific to your

driver before this file will assemble properly.

The driver header file assumes the existence of the

following functions (stubs are provided in the DRIVER.LIB library

file), which perform the various operations described in the

Installable Device Drivers chapter of the DOS Reference Manual:

Init()

MediaCheck()

BuildBPB()

IoCtlIn()

Input()

ndInput()

InputStatus()

InputFlush()

Output()

OutVerify()

OutStatus()

OutFlush()

IoCtlOut()

DevOpen()

DevClose()

RemMedia()

A sample character device driver is included in this

package, which allows for a device named "MON".  Outputting to

this device will cause characters to appear on the IBMPC

monochrome screen.  Study of the file MONO.C should reveal the

details of this implementation.

Questions or comments on this package may be directed to:

Frank Whaley

7211 Camino Colegio

Rohnert Park, CA  94928

Through the balance of this document, the pseudo-type "Addr"

is used to describe a long integer value (32-bits) which is used

to contain a memory address.  Note that the Segment:Offset

addressing method used in the 8086 family of processors causes

this data type to be incompatible with true long integers.  The

following definition is assumed:

typedef   long Addr;

Addr EndAddr();

EndAddr() returns the driver's ending address, as required

by the Init function call.

Addr Dword(ptr)

char    *ptr;

Dword() converts a 16-bit pointer (into the driver's data

segment) to a 32-bit pointer.

int CopyB(from, to, len)

Addr    from,

to;

int     len;

CopyB() copies a section of memory "len" bytes in length

from the area whose address is "from" to the area whose

address is "to".  Note that this code assumes that the

memory sections do not overlap and that a forward copy is

correct.  This function returns the number of bytes copied.

int CopyW(from, to, len)

Addr    from,

to;

int     len;

CopyW() copies a section of memory "len" words in length

from the area whose address is "from" to the area whose

address is "to".  Note that this code assumes that the

memory sections do not overlap and that a forward copy is

correct.  This function returns the number of words copied.

char InB(port)

int     port;

InB() inputs and returns a byte from the hardware port

described by "port".

int InW(port)

int     port;

InW() inputs and returns a word from the hardware port

described by "port".

char OutB(byte, port)

char    byte;

int     port;

OutB() outputs the byte "byte" to the hardware port

described by "port".  This function returns the output byte.

char OutW(word, port)

int     word,

port;

OutW() outputs the word "word" to the hardware port

described by "port".  This function returns the output word.

char PeekB(addr)

Addr    addr;

PeekB() returns the byte value found at the absolute address

given by "addr".

int PeekW(addr)

Addr    addr;

PeekW() returns the word value found at the absolute address

given by "addr".

char PokeB(val, addr)

char    val;

Addr    addr;

PokeB() stores the value "val" into the absolute byte

address given by "addr".  This function returns "val".

int PokeB(val, addr)

int     val;

Addr    addr;

PokeB() stores the value "val" into the absolute word

address given by "addr".  This function returns "val".

int SetB(start, len, byte)

Addr    start;

int     len;

char    byte;

SetB() initializes a section of memory "len" bytes in

length, beginning with the address given by "start", to the

value "byte".  This function returns the number of bytes

set.

int SetW(start, len, word)

Addr    start;

int     len,

word;

SetW() initializes a section of memory "len" words in

length, beginning with the address given by "start", to the

value "word".  This function returns the number of words

set.